Business Requirement Specification (BRS)

Scope For SalesCom Automation Project

Zabbix Monitoring & Grafana Dashboard Integration

| Project / Service Name | SalesCom Automation Project |
| --- | --- |
| Document Title | Scope For SalesCom Automation Project in Grafana Dashboard |
| BRS Document Version | Version 1.1 |
| Date | 23 June 2026 |
| Requestor Name | Md. Maruf Ul Alam |
| Requestor Email | maruf.alam@banglalink.net |
| Requestor Phone | 01962424434 |
| Requestor Team Name | Core Business Application and Service Lead Engineer, Business Support System |
| Document Approved By | TBD |
| Document Approved Date | TBD |

# 1. Overview

The SalesCom Automation Project is a sales-com automation platform hosted across the Gazipur (GZ) and Comilla (CM) data centers. The platform runs on a set of application servers and a supporting database tier. To ensure service availability, performance and timely fault detection, the platform must be continuously monitored through Zabbix and visualized through the Grafana dashboard, in line with the existing Sales Solution System dashboards (Retailer, Biometric, RSO, POS, DMS).

This BRS defines the monitoring scope for the SalesCom Automation Project, covering infrastructure health, application log (file) processing and application-level process monitoring. The objective is to give the operations and NOC teams a single, real-time view of platform health, API performance and back-end processing (DAG, Report Generation and EV processing), with D-0 / D-1 / D-7 comparison for trend analysis.

# 2. Scope of the Project

The SalesCom Automation platform consists of 15 hosts in total, distributed as follows:

- Application Servers - Gazipur (GZ): 4 hosts

- Application Servers - Comilla (CM): 5 hosts (3 IP addresses confirmed, 2 pending)

- Database VM (DB VM): 3 hosts (IP addresses pending)

- Database (DB): 3 hosts (1 test DB IP confirmed, 2 pending)

These hosts will be monitored through three methods:

- Zabbix Agent - infrastructure / resource utilisation (CPU, Memory, Storage).

- File Processing - Nginx access-log based API performance metrics.

- Zabbix Trapper - application-level process counters forwarded on an hourly basis (DAG, Report Generation, EV processing).

All graphs will follow the existing Sales Solution System dashboard UX. As with the current dashboards, Grafana should delay the last 5 minutes of records for proper visibility, and KPI data retention will be 8 days.

Server specification: every host runs Linux with 15 GB RAM and 8 vCPU cores. Logging vs metrics: SalesCom application logs are produced via Serilog, stored in Loki and viewed/filtered in Grafana (Prometheus is not used). All infrastructure, directory-wise disk, Docker-container and application-process metrics in this BRS are collected by Zabbix (Agent2 / Trapper / file-processing) and visualized in the same Grafana, with triggers routed to UMS.

# 3. Host List & IP Details

The following table lists the SalesCom Automation Project hosts to be integrated with Zabbix, in the format required by the Platform team, along with their IP addresses. Host names follow the Banglalink naming convention. For the complete Platform Integration request format, please refer to the companion Excel workbook.

| S.L. | Host Name | IP Address | Location | Category / Status |
| --- | --- | --- | --- | --- |
| 1 | GZVLSALCOMAPP01 | TBD | Gazipur (GZ) | App Server (TBD) |
| 2 | GZVLSALCOMAPP02 | TBD | Gazipur (GZ) | App Server (TBD) |
| 3 | GZVLSALCOMAI01 | TBD | Gazipur (GZ) | App Server (TBD) |
| 4 | GZVLSALCOMAI02 | TBD | Gazipur (GZ) | App Server (TBD) |
| 5 | CMVLSALCOMAPP01 | 172.19.1.189 | Comilla (CM) | App Server (Confirmed) |
| 6 | CMVLSALCOMAPP02 | TBD | Comilla (CM) | App Server (TBD) |
| 7 | CMVLSALCOMAI01 | 172.19.1.191 | Comilla (CM) | App Server (Confirmed) |
| 8 | CMVLSALCOMAI02 | TBD | Comilla (CM) | App Server (TBD) |
| 9 | CMVLSALCOMAITEST01 | 172.19.1.193 | Comilla (CM) | App Server - Master Test (Confirmed) |
| 10 | CMVLSALCOMDBVM01 | TBD | Comilla (CM) | DB VM (TBD) |
| 11 | CMVLSALCOMDBVM02 | TBD | Comilla (CM) | DB VM (TBD) |
| 12 | CMVLSALCOMDBVM03 | TBD | Comilla (CM) | DB VM (TBD) |
| 13 | CMVLSALCOMDB01 | TBD | Comilla (CM) | Database (TBD) |
| 14 | CMVLSALCOMDB02 | TBD | Comilla (CM) | Database (TBD) |
| 15 | CMVLSALCOMDBTEST01 | 172.19.51.30 | Comilla (CM) | Database - Test DB (Confirmed) |

# 4. Monitoring Requirements

## 4.1  Zabbix Agent Monitoring

Infrastructure / resource utilization will be collected through the Zabbix Agent installed on every server host (9 application servers and 3 Database VMs). The requestor confirms that Zabbix host agent HA configuration is available on all hosts. Each metric is to be visualized with D-0 and D-1 comparison.

| S.L. | Item (Metric) | Monitoring Method | Comparison | Dashboard Modality |
| --- | --- | --- | --- | --- |
| 1 | CPU Utilization (%) | Zabbix Agent | D-0, D-1 | Line graph |
| 2 | Memory Utilization (%) | Zabbix Agent | D-0, D-1 | Line graph |
| 3 | Storage Utilization (%) | Zabbix Agent | D-0, D-1 | Line graph |
| 4 | Directory-wise Disk Usage (bytes) | Zabbix Agent2 (vfs.dir.size) | D-0, D-1 | Line graph |

Directory-wise disk usage is collected via Zabbix Agent2 (vfs.dir.size). Watched directories — App/AI hosts: application log dir, /var/lib/docker, SeaweedFS volume dir, upload-staging dir; DB hosts: PostgreSQL data and WAL dir. A longer (10-minute) interval is used because the item walks the directory tree.

Applicable hosts: GZ App (4) + CM App (5) + DB VM (3) = 12 server hosts. Each host is shown on the same panel with a distinct color code for easier visual comparison.

## 4.2  File Processing Monitoring (Nginx Log)

API performance metrics are derived from Nginx access logs through the file-processing (big-data) pipeline, with a 1-minute sync interval and the existing log format used by the Retailer App. Each metric is to be visualized with D-0, D-1 and D-7 comparison. This applies to the 4 application servers (GZ App 2 + CM App 2).

| S.L. | Item (Metric) | Monitoring Method | Comparison | Dashboard Modality |
| --- | --- | --- | --- | --- |
| 1 | Nginx Hit Count | File Processing | D-0, D-1, D-7 | Line graph |
| 2 | Average Response Time | File Processing | D-0, D-1, D-7 | Line graph |
| 3 | Maximum Response Time | File Processing | D-0, D-1, D-7 | Line graph |
| 4 | Error Count | File Processing | D-0, D-1, D-7 | Line graph |
| 5 | Success Percentage (%) | File Processing | D-0, D-1, D-7 | Stat / Line graph |

Success Percentage logic: Success % = (count of successful requests, HTTP status 200) / (total request count) x 100, computed across the SalesCom application servers, consistent with the Retailer App calculation.

## 4.3  Zabbix Trapper Monitoring (Hourly Basis)

Application-level process counters are forwarded to Zabbix via Trapper on an hourly basis (60-minute interval). The application will send trapper values to the Zabbix trapper servers (172.16.7.221 and 172.16.7.196) using the item keys below. Each counter is to be visualized with D-0, D-1 and D-7 comparison. Three process groups are in scope: DAG Process, Report Generation and EV Processing.

a) DAG Process Count

| S.L. | Counter | Trapper Item Key | Value Type | Interval | Comparison |
| --- | --- | --- | --- | --- | --- |
| 1 | Pass Count | salcom.dag.pass | Numeric | 60 min | D-0,1,7 |
| 2 | Fail Count | salcom.dag.fail | Numeric | 60 min | D-0,1,7 |

b) Report Generation

| S.L. | Counter | Trapper Item Key | Value Type | Interval | Comparison |
| --- | --- | --- | --- | --- | --- |
| 1 | Generated Count | salcom.report.generated | Numeric | 60 min | D-0,1,7 |
| 2 | Failed Count | salcom.report.failed | Numeric | 60 min | D-0,1,7 |
| 3 | Missed Count | salcom.report.missed | Numeric | 60 min | D-0,1,7 |

c) EV Processing

| S.L. | Counter | Trapper Item Key | Value Type | Interval | Comparison |
| --- | --- | --- | --- | --- | --- |
| 1 | EV Success Count | salcom.ev.success | Numeric | 60 min | D-0,1,7 |
| 2 | EV Error Count | salcom.ev.error | Numeric | 60 min | D-0,1,7 |

Note: Trapper item keys are suggested values and may be aligned to the application's actual sender configuration. The sending host name will be confirmed by the SalesCom application team.

## 4.4  Docker Container Monitoring

Container-level health and resource utilization are collected through the built-in "Docker by Zabbix agent 2" template (the agent2 docker plugin reads /var/run/docker.sock). Containers are auto-discovered via Zabbix low-level discovery: web, api, sql-gen, executor, ev-worker, rabbitmq, seaweedfs, loki, grafana. This applies to the nine application/AI servers that run Docker (GZ App 4 + CM App 5); the bare-metal database hosts run no containers. Each metric is visualized with D-0 / D-1 comparison; container-down and daemon-down conditions raise triggers routed to UMS.

| S.L. | Item (Metric) | Monitoring Method (Item Key, Agent2) | Interval | Comparison | Trigger (knock → UMS) |
| --- | --- | --- | --- | --- | --- |
| 1 | Docker Daemon Up (Ping) | docker.ping | 1 min | D-0 | Critical if 0 (daemon down) |
| 2 | Running Container Count | docker.containers | 5 min | D-0, D-1 | - |
| 3 | Image Count | docker.images | 5 min | D-0 | - |
| 4 | Container Up/Running Status | docker.container_info[<name>,State] | 1 min | - | Critical if State != running |
| 5 | Container Health | docker.container_info[<name>,Health] | 1 min | - | High if unhealthy |
| 6 | Container CPU Utilization (%) | docker.container_stats[<name>] (cpu) | 5 min | D-0, D-1 | - |
| 7 | Container Memory Used (%) | docker.container_stats[<name>] (mem / limit) | 5 min | D-0, D-1 | High if > 90% of limit |

# 5. Grafana Dashboard Requirements

The Grafana dashboard for SalesCom Automation should follow the generic Sales Solution System UX, similar to the Retailer app dashboards. Requirements:

- Panels: Separate panels for Infrastructure (CPU, Memory, Storage), API Performance (Hit, Average, Max, Error, Success %) and Application Process (DAG, Report Generation, EV).

- Multi-host view: All application servers shown on the same panel with distinct colour codes; each panel takes the full monitor width for better visualisation.

- Comparison: D-0 and D-1 for infrastructure; D-0, D-1 and D-7 for file-processing and trapper metrics.

- Modality: Line graph for time-series metrics; Stat/Single-value for Success % and total counts; Table where a per-host/per-counter breakdown is required.

- Data delay: Grafana should delay the last 5 minutes of records for proper visibility.

- KPI retention: Data retention will be 8 days.

- NOC monitoring: Yes.

- Threshold / legend: Shared in excel file.

- Dashboard access: Yes and shared in excel file.

## 5.1  Suggested Panel Layout (Box Representation)

Indicative dashboard layout for SalesCom Automation:

| Section | Panels | Source | Comparison |
| --- | --- | --- | --- |
| Infrastructure | CPU %, Memory %, Storage % (per host) | Zabbix Agent | D-0, D-1 |
| API Performance | Total Hit, Hit/Avg/Max per server, Error Count, Success % | File Processing | D-0, D-1, D-7 |
| DAG Process | Pass Count, Fail Count | Trapper | D-0, D-1, D-7 |
| Report Generation | Generated, Failed, Missed | Trapper | D-0, D-1, D-7 |
| EV Processing | EV Success, EV Error | Trapper | D-0, D-1, D-7 |
| Infrastructure (Directory) | Directory-wise Disk Usage (per watched path) | Zabbix Agent2 | D-0, D-1 |
| Docker Containers | Container Up/Down, CPU %, Memory %, Container & Image count | Zabbix Agent2 (Docker) | D-0, D-1 |